# Enterprise LAN with VLAN Configuration

## 📌 Project Overview

Implementation of an enterprise LAN using VLAN segmentation for three departments (IT, Sales, and Support) with inter-VLAN routing and internet connectivity via NAT.

## 🗺️ Network Topology

Internet (Cloud-PT Cloud0)

|

[Router0 - 2911]

|

802.1Q Trunk

|

[Switch0 - 2960-24TT]

|

┌────┴────┬────────┐

|         |        |

VLAN 10  VLAN 20  VLAN 30

(IT-Red) (Sales)  (Support)

|         |        |

2 PCs    2 PCs    2 PCs

## 📊 VLAN Design

| VLAN | Department | Network | Gateway | Devices |
| --- | --- | --- | --- | --- |
| 10 | IT | 192.168.10.0/24 | 192.168.10.1 | PC1, PC0 |
| 20 | Sales | 192.168.20.0/24 | 192.168.20.1 | PC2, PC3 |
| 30 | Support | 192.168.30.0/24 | 192.168.30.1 | PC5, PC4 |

## ⚙️ Configuration Steps

### Step 1: Switch Configuration

Create VLANs:

Switch(config)# vlan 10

Switch(config-vlan)# name IT-Department

Switch(config)# vlan 20

Switch(config-vlan)# name Sales-Department

Switch(config)# vlan 30

Switch(config-vlan)# name Support-Department

Assign Access Ports:

! IT Department (VLAN 10)

Switch(config)# interface range fa0/1-5

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

! Sales Department (VLAN 20)

Switch(config)# interface range fa0/6-10

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

! Support Department (VLAN 30)

Switch(config)# interface range fa0/11-15

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 30

Configure Trunk:

Switch(config)# interface gig0/1

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk allowed vlan 10,20,30

### Step 2: Router Configuration (Router-on-a-Stick)

Enable Physical Interface:

Router(config)# interface g0/0

Router(config-if)# no shutdown

Configure Subinterfaces:

! VLAN 10 - IT

Router(config)# interface g0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 192.168.10.1 255.255.255.0

! VLAN 20 - Sales

Router(config)# interface g0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 192.168.20.1 255.255.255.0

! VLAN 30 - Support

Router(config)# interface g0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 192.168.30.1 255.255.255.0

Configure NAT for Internet:

! WAN Interface

Router(config)# interface g0/1

Router(config-if)# ip address dhcp

Router(config-if)# ip nat outside

Router(config-if)# no shutdown

! Inside Interfaces

Router(config)# interface g0/0.10

Router(config-subif)# ip nat inside

Router(config)# interface g0/0.20

Router(config-subif)# ip nat inside

Router(config)# interface g0/0.30

Router(config-subif)# ip nat inside

! NAT Configuration

Router(config)# access-list 1 permit 192.168.10.0 0.0.0.255

Router(config)# access-list 1 permit 192.168.20.0 0.0.0.255

Router(config)# access-list 1 permit 192.168.30.0 0.0.0.255

Router(config)# ip nat inside source list 1 interface g0/1 overload

Router(config)# ip route 0.0.0.0 0.0.0.0 g0/1

### Step 3: DHCP Configuration on Router

Configure DHCP Pools for Each VLAN:

! Exclude gateway addresses

Router(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.10

Router(config)# ip dhcp excluded-address 192.168.20.1 192.168.20.10

Router(config)# ip dhcp excluded-address 192.168.30.1 192.168.30.10

! DHCP Pool for IT Department (VLAN 10)

Router(config)# ip dhcp pool IT-POOL

Router(dhcp-config)# network 192.168.10.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.10.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

! DHCP Pool for Sales Department (VLAN 20)

Router(config)# ip dhcp pool SALES-POOL

Router(dhcp-config)# network 192.168.20.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.20.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

! DHCP Pool for Support Department (VLAN 30)

Router(config)# ip dhcp pool SUPPORT-POOL

Router(dhcp-config)# network 192.168.30.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.30.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

Verify DHCP Configuration:

Router# show ip dhcp pool

Router# show ip dhcp binding

### Step 4: PC Configuration (DHCP)

IT Department (VLAN 10):

- PC1: Set to DHCP (receives 192.168.10.11)

- PC0: Set to DHCP (receives 192.168.10.12)

Sales Department (VLAN 20):

- PC2: Set to DHCP (receives 192.168.20.11)

- PC3: Set to DHCP (receives 192.168.20.12)

Support Department (VLAN 30):

- PC5: Set to DHCP (receives 192.168.30.11)

- PC4: Set to DHCP (receives 192.168.30.12)

On Each PC:

- Click PC → Desktop → IP Configuration

- Select DHCP (instead of Static)

- PC automatically receives IP, Subnet Mask, Gateway, and DNS

- Verify using ipconfig command

## 🧪 Verification

Check VLANs:

Switch# show vlan brief

Switch# show interfaces trunk

Check Router:

Router# show ip interface brief

Router# show ip route

Router# show ip nat translations

Test Connectivity:

ping 192.168.10.1    (from IT PC to gateway)

ping 192.168.20.10   (from IT PC to Sales PC - inter-VLAN)

ping 8.8.8.8         (internet connectivity)

## 🔍 Common Issues

No Inter-VLAN Communication:

- Check trunk: show interfaces trunk

- Verify subinterfaces: show ip interface brief

- Ensure physical interface is up: no shutdown on g0/0

No Internet Access:

- Check NAT: show ip nat translations

- Verify default route: show ip route

- Confirm WAN interface is up

## 🎯 Learning Outcomes

✓ VLAN configuration and segmentation
✓ 802.1Q trunk configuration
✓ Router-on-a-Stick inter-VLAN routing
✓ NAT configuration for internet access
✓ IP addressing and gateway setup
✓ Network verification using Cisco IOS commands

## 📝 Summary

Network Components:

- 1 Router (2911) - Inter-VLAN routing + NAT

- 1 Switch (2960-24TT) - VLAN switching

- 3 VLANs (IT, Sales, Support)

- 6 PCs across departments

- Internet connectivity via Cloud

Key Technologies:

- VLANs (10, 20, 30)

- IEEE 802.1Q Trunking

- Router-on-a-Stick

- NAT Overload (PAT)

- IPv4 Addressing